# Rowing Sensor PWA — Structure & Limitations

Summary for integrating a phone-based Progressive Web App (PWA) with the existing traccar-overlay project and Traccar GPS backend.

Repository: https://github.com/JohnSt-AHD/rowing_app
Existing host project: traccar-overlay (maps, live race, RowSafe)
Date: May 2026

# 1. Purpose

Record heart rate (HR), GPS, and accelerometer data on athletes’ phones during rowing sessions, and send that data into the existing RNZ tracking stack so traccar-overlay can display live position and related telemetry.

# 2. Recommended architecture

Treat the solution as two products: rowing_app (phone recorder) and traccar-overlay (display and operations). Do not merge them into one repo.

## 2.1 Data flow

— Phone PWA captures sensors, buffers offline in IndexedDB, uploads when online.

— GPS positions sent to Traccar via OsmAnd HTTP protocol (port 5055). Extra fields (e.g. HR) can be attached as position attributes.

— HR and high-rate accelerometer sent to a small ingest API (Vercel), stored as time-series batches keyed by device/session.

— traccar-overlay continues to poll Traccar for positions; optionally extended later to read ingest API for HR/accel.

## 2.2 Suggested repository structure (rowing_app)

rowing_app/
  apps/recorder-pwa/     — Vite + TypeScript PWA (manifest, service worker)
    src/sensors/         — gps, motion, heart-rate (Web Bluetooth)
    src/session/         — recorder, IndexedDB outbox
    src/upload/          — traccar (OsmAnd), telemetry-api
  api/                   — Vercel serverless ingest (optional phase 2)
  packages/telemetry-types/ — shared JSON/TypeScript schemas
  docs/                  — architecture and integration notes

## 2.3 Deployment

— Vercel: static PWA + api/ingest

— traccar-overlay stays on existing Vercel deployment

— Traccar admin credentials remain server-side only; phones use device IDs and session tokens

# 3. GPS update rates and comparison

## 3.1 Three separate layers

| Layer | Typical rate | Notes |
| --- | --- | --- |
| Phone → Traccar | 0.3–1 Hz (aggressive Client) | OS and app settings dominate |
| Traccar database | Same as ingest | Optional filter.minPeriod can drop fixes |
| traccar-overlay map UI | 0.03–0.33 Hz (3–10 s poll) | User setting; default 10 s |

## 3.2 Traccar Client (native app)

Interval and Distance use OR logic — reports when either threshold is met.

Fastest interval caps maximum send rate; Highest accuracy keeps GPS on for better consistency.

Practical live tracking: about 1 Hz (e.g. Interval 1–3 s, low distance).

Fleet/safety style: every 30–60 s or every 50–100 m.

Strong background tracking on Android (wake lock); iOS still subject to OS limits.

## 3.3 PWA (browser)

Geolocation watchPosition has no fixed Hz; browser may rate-limit and uses “significant change” logic.

Foreground (screen on, active tab): roughly 0.2–2 Hz, device-dependent.

Background / locked screen: often sparse or no updates — especially on iOS.

Generally underperforms Traccar Client for continuous track density.

## 3.4 traccar-overlay display

Map refresh configurable from 3 s to 60 s (default 10 s) — independent of GPS ingest rate.

Even 1 Hz stored in Traccar may appear smoother only if overlay poll is set to 3–5 s during live races.

# 4. Sensor limitations

## 4.1 GPS

PWA GPS is suitable for active recording sessions with the app in the foreground. For all-day or pocket tracking, the official Traccar Client app is more reliable.

## 4.2 Accelerometer

DeviceMotion API works over HTTPS; iOS may require explicit permission. Sample at 10–25 Hz for general motion; 50 Hz only if needed for stroke analysis, with batched uploads via the ingest API (not Traccar positions).

## 4.3 Heart rate

Web Bluetooth (GATT heart_rate service) works well on Android Chrome. iOS Safari does not support Web Bluetooth natively; alternatives include Android devices, BLE-capable browsers, or third-party iOS extensions/polyfills (extra user setup). Connection must be started from a user tap (button), not on page load.

# 5. Implementation phases

1. Scaffold Vite PWA, manifest, service worker, IndexedDB buffer.

2. GPS only → Traccar OsmAnd; verify on traccar-overlay live map.

3. Add accelerometer + ingest API.

4. Add heart rate (Android first; document iOS path).

5. Session UX: start/stop, reconnect BLE, export for debugging.

# 6. Practical recommendation for RNZ rowing

— Use Traccar Client or foreground PWA for GPS at 1–3 s intervals.

— Set traccar-overlay map refresh to 3–5 s during live events.

— Route HR and accelerometer through a dedicated ingest API, not high-rate Traccar positions.

# 7. References

GitHub repository: https://github.com/JohnSt-AHD/rowing_app

Traccar OsmAnd protocol: https://www.traccar.org/osmand/

Traccar Client configuration: https://www.traccar.org/client-configuration/

Traccar server filters: https://www.traccar.org/configuration-file/

W3C Geolocation API: https://www.w3.org/TR/geolocation/